Mahajal IoT Dashboard

Technical Data Schema & API Guide

# 1. Global Platform Configuration

This technical guide details the database schemas and available REST API endpoints for the Mahajal IoT platform. It is compiled for external development teams (such as the CM Dashboard team) to facilitate integration.

## Authentication Protocols

The application utilizes a dual-authentication mechanism. Access is granted via either Session Authentication or a Static API Key.

### API Key Header Authentication

For server-to-server endpoints (such as bulk integration pulls), third-party requests must include one of the following HTTP headers:

•  Header Key: X-External-Proxy-Key or X-API-Key
•  Header Value: MAHAJAL_IOT_SECURE_KEY_25 or h

### Session Authentication

User-facing and administrative portals require active login sessions:

•  Normal User Authorization: Checks if userId is present in the session.
•  Administrator Authorization: Checks if the logged-in user holds the admin role.

## Server Connection Details

•  Base URL (Production): https://mahajal-iot.mjp.maharashtra.gov.in
•  Base URL (Internal): https://dashboard1.mahajaliot.in
•  Default Content-Type: application/json

# 2. Database Schema Map

The platform database runs on PostgreSQL. Below is the mapping of all tables, columns, and data types defined in the source code.

### A. Region Table (region)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| region_id | Serial (PK) | Unique region identifier |
| region_name | Text (Unique) | Name of the region (e.g. Pune, Nashik, Konkan) |
| total_esr_integrated | Integer | Total number of ESRs integrated in the region |
| fully_completed_esr | Integer | Total number of fully completed ESRs |
| partial_esr | Integer | Total number of partially completed ESRs |
| total_villages_integrated | Integer | Total integrated villages count |
| fully_completed_villages | Integer | Total fully completed villages count |
| total_schemes_integrated | Integer | Total integrated schemes count |
| fully_completed_schemes | Integer | Total fully completed schemes count |
| flow_meter_integrated | Integer | Total integrated flowmeters count |
| rca_integrated | Integer | Total integrated Residual Chlorine Analyzers (RCA) count |
| pressure_transmitter_integrated | Integer | Total integrated pressure transmitters count |

### B. Scheme Status Table (scheme_status)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| sr_no | Integer | Serial number (Unique identifier representation) |
| scheme_id | Text | Official scheme identifier |
| region | Text | Region name |
| circle | Text | Circle name |
| division | Text | Division name |
| sub_division | Text | Sub-division name |
| block | Text | Taluka / Block name |
| scheme_name | Text | Name of the water supply scheme |
| agency | Text | Managing agency |
| number_of_village | Integer | Total number of villages under the scheme |
| total_villages_integrated | Integer | Integrated villages count on IoT |
| no_of_functional_village | Integer | Functional villages count |
| no_of_partial_village | Integer | Partially functional villages count |
| no_of_non_functional_village | Integer | Non-functional villages count |
| fully_completed_villages | Integer | Fully completed villages count |
| total_number_of_esr | Integer | Total ESRs under the scheme |
| scheme_functional_status | Text | Current operational status of the scheme |
| total_esr_integrated | Integer | Total ESRs integrated on IoT |
| no_fully_completed_esr | Integer | Fully completed ESR count |
| balance_to_complete_esr | Integer | Remaining ESRs to integrate |
| flow_meters_connected | Integer | Connected flow meters count |
| pressure_transmitter_connected | Integer | Connected pressure transmitters count |
| residual_chlorine_analyzer_connected | Integer | Connected RCAs count |
| fully_completion_scheme_status | Text | Overall physical progress status |
| mjp_commissioned | Text | Commissioned by MJP ('Yes' / 'No') |
| mjp_fully_completed | Text | MJP completion status ('Fully Completed' / 'In Progress') |
| water_supply | Text | Water supply active status |
| agency_type | Text | JJM Agency type: MJP or ZP |
| water_supply_status | Text | Categorized: 'Full', 'Partial', or 'No' |
| dashboard_url | Text | URL of the PI Vision dashboard for this scheme |

### C. Water Scheme Data Table (water_scheme_data)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| region / circle / division / sub_division / block | Text | Geographical hierarchy |
| scheme_id | Varchar(100) | Part of composite PK |
| scheme_name | Text | Name of the scheme |
| village_name | Text | Part of composite PK |
| population | Integer | Total population of the village |
| number_of_esr | Integer | Total number of ESRs in this village |
| water_value_day1 to day7 | Decimal | Daily water supply volume logs for the past 7 days |
| lpcd_value_day1 to day7 | Decimal | Daily calculated Liters Per Capita per Day (LPCD) |
| water_date_day1 to day7 | Varchar(20) | Measurement dates for water values |
| lpcd_date_day1 to day7 | Varchar(20) | Measurement dates for LPCD values |
| consistent_zero_lpcd_for_a_week | Integer | Flag (0/1) for zero water supply for 7 straight days |
| below_55_lpcd_count | Integer | Days with supply below the JJM standard of 55 LPCD |
| above_55_lpcd_count | Integer | Days with supply meeting/exceeding 55 LPCD |
| dashboard_url | Text | Direct link to PI Vision dashboard |

### D. Chlorine Data Table (chlorine_data)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| scheme_id / village_name / esr_name | Varchar(100/255) | Composite Primary Key identifying each ESR |
| region / circle / division / sub_division / block | Varchar(100/255) | Geographical fields |
| chlorine_value_1 to chlorine_value_7 | Decimal | Residual Chlorine values (mg/l) for past 7 days |
| chlorine_date_day_1 to day_7 | Varchar(15) | Dates corresponding to the chlorine logs |
| number_of_consistent_zero_value_in_chlorine | Integer | Days with zero chlorine readings |
| chlorine_less_than_02_mgl | Decimal | Calculated value for chlorine below 0.2 mg/l |
| chlorine_between_02_05_mgl | Decimal | Calculated value for chlorine between 0.2 and 0.5 mg/l |
| chlorine_greater_than_05_mgl | Decimal | Calculated value for chlorine above 0.5 mg/l |
| dashboard_url | Text | Direct link to PI Vision dashboard |

### E. Pressure Data Table (pressure_data)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| scheme_id / village_name / esr_name | Text | Composite Primary Key identifying each ESR |
| pressure_value_1 to pressure_value_7 | Decimal | Daily pressure measurements (bar) for 7 days |
| pressure_date_day_1 to day_7 | Varchar(15) | Dates corresponding to pressure logs |
| number_of_consistent_zero_value_in_pressure | Integer | Count of days with zero pressure |
| pressure_less_than_02_bar | Decimal | Days with pressure below 0.2 bar |
| pressure_between_02_07_bar | Decimal | Days with pressure between 0.2 and 0.7 bar |
| pressure_greater_than_07_bar | Decimal | Days with pressure above 0.7 bar |
| dashboard_url | Text | Link to PI Vision ESR dashboard |

### F. Water Consumption Table (water_consumption)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| scheme_id / village_name / esr_name | Varchar(50/255) | Composite Primary Key identifying each ESR |
| flow_rate_m3 | Decimal | Flow rate reading in cubic meters |
| flow_meter_connected | Varchar(50) | Hardware connection status (Yes/No) |
| online_status | Varchar(20) | Sensor live connection state (online/offline) |
| time_duration | Varchar(50) | Pumping duration |
| esr_capacity | Decimal | Storage capacity of the ESR |
| water_value_day1 to day7 | Decimal | Rolling daily water consumption values (liters) |
| water_date_day1 to day7 | Varchar(15) | Dates corresponding to water logs |
| consistent_zero_consumption | Integer | Flag for zero consumption across the week |
| percentage_consumption_previous_day | Decimal | Delta compared to previous day |
| dashboard_url | Text | Link to PI Vision ESR dashboard |

## 2.3 Permanent Historical Storage Tables

These tables store raw historical logs uploaded from CSV/Excel sheets or aggregated on a daily basis.

### G. Water Scheme Data History Table (water_scheme_data_history)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| id | Serial (PK) | Unique row identifier |
| region / circle / division / sub_division / block | Varchar(100) | Geographical hierarchy fields |
| scheme_id | Varchar(100) | Water scheme identifier |
| scheme_name | Varchar(255) | Name of the scheme |
| village_name | Varchar(255) | Name of the village |
| population | Integer | Village population |
| number_of_esr | Integer | Total ESRs count in the village |
| data_date | Varchar(15) (Not Null) | Log date (e.g. DD-MM-YYYY) |
| water_value | Decimal | Total water volume supplied (liters) |
| lpcd_value | Decimal | Calculated LPCD |
| uploaded_at | Timestamp | System insertion time |
| upload_batch_id | Varchar(50) | Groups records uploaded in the same batch/file |
| dashboard_url | Text | Direct link to PI Vision dashboard |

### H. Scheme LPCD Data History Table (scheme_lpcd_data_history)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| id | Serial (PK) | Unique row identifier |
| region / circle / division / sub_division / block | Varchar(100) | Geographical hierarchy fields |
| scheme_id | Varchar(100) | Water scheme identifier |
| scheme_name | Varchar(255) | Name of the scheme |
| total_population | Integer | Total scheme population |
| total_villages | Integer | Total villages count under the scheme |
| villages_below_55 | Integer | Villages with LPCD under 55 |
| villages_above_55 | Integer | Villages with LPCD at or above 55 |
| villages_zero_supply | Integer | Villages with zero water supply |
| data_date | Varchar(15) (Not Null) | Log date (e.g. DD-MM-YYYY) |
| water_value | Decimal | Total water volume supplied for the scheme |
| lpcd_value | Decimal | Scheme-level LPCD |
| uploaded_at | Timestamp | System insertion time |
| upload_batch_id | Varchar(50) | Groups records uploaded in the same batch/file |
| dashboard_url | Text | Direct link to PI Vision dashboard |
| mjp_commissioned | Text | Commission status flag |

### I. Chlorine History Table (chlorine_history)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| id | Serial (PK) | Unique row identifier |
| region / circle / division / sub_division / block | Varchar(100) | Geographical hierarchy fields |
| scheme_id | Varchar(100) | Water scheme identifier |
| scheme_name | Varchar(255) | Name of the scheme |
| village_name | Varchar(255) | Name of the village |
| esr_name | Varchar(255) | Name of the ESR/Reservoir |
| chlorine_date | Varchar(15) (Not Null) | Log date (e.g. DD-MM-YYYY) |
| chlorine_value | Decimal | Residual Chlorine level (mg/l) |
| uploaded_at | Timestamp | System insertion time |
| upload_batch_id | Varchar(50) | Groups records uploaded in the same batch/file |
| dashboard_url | Text | Direct link to PI Vision dashboard |

### J. Pressure History Table (pressure_history)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| id | Serial (PK) | Unique row identifier |
| region / circle / division / sub_division / block | Varchar(100) | Geographical hierarchy fields |
| scheme_id | Varchar(100) | Water scheme identifier |
| scheme_name | Varchar(255) | Name of the scheme |
| village_name | Varchar(255) | Name of the village |
| esr_name | Varchar(255) | Name of the ESR/Reservoir |
| pressure_date | Varchar(15) (Not Null) | Log date (e.g. DD-MM-YYYY) |
| pressure_value | Decimal | Water pressure value (bar) |
| uploaded_at | Timestamp | System insertion time |
| upload_batch_id | Varchar(50) | Groups records uploaded in the same batch/file |
| dashboard_url | Text | Direct link to PI Vision dashboard |

### K. Water Consumption History Table (water_consumption_history)

| Column Name | Data Type | Description / Notes |
| --- | --- | --- |
| id | Serial (PK) | Unique row identifier |
| region / circle / division / sub_division / block | Varchar(100) | Geographical hierarchy fields |
| scheme_id | Varchar(100) | Water scheme identifier |
| scheme_name | Varchar(255) | Name of the scheme |
| village_name | Varchar(255) | Name of the village |
| esr_name | Varchar(255) | Name of the ESR/Reservoir |
| data_date | Varchar(15) (Not Null) | Log date (e.g. DD-MM-YYYY) |
| water_value | Decimal | Water volume consumed (liters) |
| flow_rate_m3 | Decimal | Flow rate (m3) reading |
| esr_capacity | Decimal | Capacity of the reservoir |
| uploaded_at | Timestamp | System insertion time |
| upload_batch_id | Varchar(50) | Groups records uploaded in the same batch/file |
| dashboard_url | Text | Direct link to PI Vision dashboard |

# 3. API Endpoint Directory

This section catalogs the REST API endpoints available for third-party client integrations.

## Water Scheme & LPCD Status APIs

Root Mounting Path: /api/water-scheme-data

| Method | Endpoint | Query Params / Payload | Description |
| --- | --- | --- | --- |
| GET | / | region, minLpcd, maxLpcd, zeroSupplyForWeek, agencyType | Fetches village-level 7-day LPCD and water volumes. |
| GET | /village-counts | region, mjpCommissioned, agencyType | Returns village functional, partial, and non-functional counts. |
| GET | /village-stats | region, agencyType | Returns water metrics and rolling target achievements. |
| GET | /population-stats | region, agencyType | Retrieves populations receiving JJM standard supply (>55 LPCD). |
| GET | /population-change | region | Returns historical population tracking delta. |
| GET | /lpcd-stats | region, agencyType | Returns count of villages categorized by LPCD supply range. |
| GET | /template | None | Returns an XLSX template for uploading LPCD statistics. |
| POST | /import/excel | file (Multipart form-data) | Bulk imports LPCD spreadsheet. Syncs population tracking. |
| POST | /import/csv | file (Multipart form-data) | Bulk imports CSV records. |

## Live Sensor APIs (Current 7-Day Context)

Root Mounting Path: Multiple Paths

| Method | Endpoint | Query Params / Payload | Description |
| --- | --- | --- | --- |
| GET | /api/chlorine | region, chlorineRange | Returns rolling chlorine measurements per ESR. |
| GET | /api/chlorine/dashboard-stats | None | Returns counts of safe vs. unsafe tests. |
| GET | /api/chlorine/overall-region-comparison | None | Returns regional hardware comparison. |
| GET | /api/pressure | region | Returns current pressure values per ESR. |
| GET | /api/pressure/dashboard-stats | None | Returns counts of low/normal/high pressure points. |
| GET | /api/water-consumption | region, agencyType | Returns water consumption volume logs per ESR. |
| GET | /api/flowmeter/overall-region-comparison | None | Returns online/offline counts of flowmeters by region. |

## Historical & Bulk Download APIs

Root Mounting Path: Multiple Paths

| Method | Endpoint | Query Params / Payload | Description |
| --- | --- | --- | --- |
| GET | /api/combined-esr-download/historical | startDate, endDate, region | Returns a merged time-series table of water volume, chlorine, and pressure. |
| GET | /api/scheme-lpcd-data/history | start_date, end_date, limit | Returns bulk scheme-level historical LPCD logs. |
| GET | /api/chlorine/historical | startDate, endDate, region | Returns raw historical chlorine measurements. |
| GET | /api/pressure/historical | startDate, endDate, region | Returns raw historical pressure logs. |
| GET | /api/water-consumption/historical | startDate, endDate, region | Returns raw historical water volume consumption. |

## Helpdesk & System Operations

Root Mounting Path: /api/helpdesk

| Method | Endpoint | Query Params / Payload | Description |
| --- | --- | --- | --- |
| POST | /tickets | title, category, priority, description, attachments (Multipart) | Creates a ticket. Dispatches notifications. |
| GET | /tickets | None | Lists logged-in user's tickets. |
| GET | /tickets/:id | None | Returns details of a specific ticket with attachments. |
| GET | /admin/tickets | status, category, region, search | Admin only: Searches all tickets in the system. |
| PUT | /tickets/:id | status, admin_comments | Admin only: Updates ticket status/comments. |
| POST | /tickets/:id/resolve | admin_comments | Admin only: Resolves ticket and emails user. |
| POST | /tickets/:id/reopen | reopen_reason | User only: Reopens a resolved ticket within 48 hours. |
| POST | /tickets/:id/close | close_reason | User only: Closes a resolved ticket. |